What are MKIDs?

Microwave Kinetic Inductance Detectors (MKIDs) are superconducting photon detectors that operate at temperatures near 0.1 K. When a photon is absorbed, it breaks apart Cooper pairs - the electron pairs responsible for superconductivity - changing the kinetic inductance of the resonator. This shift is measured as a change in phase or frequency of a microwave signal, giving both the arrival time and energy of each photon.

How it works

Each MKID pixel is a superconducting resonator tuned to a unique microwave frequency. Thousands of pixels can be read out simultaneously over a single coaxial cable - a technique called frequency-domain multiplexing. This dramatically reduces the number of wires entering the cryostat and makes large-format arrays practical.

Key advantages

MKIDs are intrinsically energy-resolving: the number of broken Cooper pairs is proportional to the photon energy, so the detector acts as a spectrometer without any dispersive optics. They are also photon-counting with microsecond time resolution, and relatively straightforward to fabricate using standard thin-film deposition.

Applications

MKIDs are used across a wide range of wavelengths - from millimeter waves and infrared through optical and into X-rays. Current instruments include cosmic microwave background telescopes, direct imaging cameras for exoplanet science, and X-ray spectrometers for laboratory astrophysics. Their scalability makes them a leading technology for next-generation detector arrays requiring tens of thousands of pixels.